Safety Data Sheet for Instant Adhesive

Material Safety Data Sheet

According to 91/155 EC

Page 1/5

Printing date 31.01.2012	Reviewed on 31.01.2012

┌────────────────────────────────────────────────────────────────────────────┐

│	1 Identification of the substance/preparation and of the	│

│	company/undertaking	│

│	│

│	· Product details	│

│	│

| │ │ | · | Trade name: Instant | Adhesive |  |  |  | │ │ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| │ | · | Supplier: |  |  |  |  | │ |
| │ |  | Ultimate Dental |  |  |  |  | │ |
| │ |  | 25/632-642 Clayton Rd Cl Clayton South Vic |  |  |  |  | │ |
| │ |  | Clayton South Vic 3169 |  |  |  |  | │ |
| │ |  |  |  |  |  |  | │ |

│	   Telephone: (03)9532 1799

| Fax: | (03) 9555 9458 |
| --- | --- |

│	· Information in case of emergency: 13 11 26	│

├────────────────────────────────────────────────────────────────────────────┤

| │ | 2 | Composition/information on |  |  | ingredients |  |  |  |  | │ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| │ |  |  |  |  |  |  |  |  |  | │ |
| │ | · | Chemical characterization |  |  |  |  |  |  |  | │ |
| │ | · | Description: |  |  |  |  |  |  |  | │ |
| │ |  | Mixture of substances |  | listed |  | below | with | non-hazardous | additions. | │ |
| │ |  |  |  |  |  |  |  |  |  | │ |
| │ |  | Dangerous | components: |  |  |  |  |  |  | │ |
| │ |  | 7085-85-0 | ethyl 2-cyanoacrylate |  |  |  |  |  |  | >30 % │ |
| │ |  |  | Xi; R 36/37/38 |  |  |  |  |  |  | │ |

├────────────────────────────────────────────────────────────────────────────┤

│	3 Hazards identification	│

│	│

│	· Hazard description: Xi Irritant	│

│	· Information concerning to particular hazards to man and environment:	│

│	R 36/37/38 Irritating to eyes, respiratory system and skin.	│

│	Cyanacrylate - DANGER! Glues skin and eyelids within seconds. Should be │

│	kept out of children's reach.	│

│	· Classification system:	│

│	The classification is according to the latest editions of the EU-lists, │

│	and extended by company and literature data.	│

├────────────────────────────────────────────────────────────────────────────┤

│	4 First-aid measures	│

│	│

│	· General information:	│

│	Immediately remove any clothing soiled by the product.	│

│	· After inhalation:	│

│	Supply fresh air. If required, provide artificial respiration. Keep	│

│	patient warm. Consult doctor if symptoms persist.	│

│	In case of unconsciousness place patient stably in side position for	│

│	transportation.	│

│	· After skin contact:	│

│	After contact with skin, wash immediately with plenty of soap and	│

│	water.	│

│	· After eye contact:	│

│	In case of contact with eyes, rinse immediately with plenty of water	│

│	and seek medical advice.	│

│	· After swallowing:	│

│	Rinse out mouth and then drink plenty of water.	│

│	If swallowed, do not induce vomiting: seek medical advice immediately	│

│	and show this container or label.	│

│	· Information for doctor:	│

│	· The following symptoms may occur: Breathing difficulty	│

│	(Contd. on page 2) │

Safety Data Sheet for Instant Adhesive

Material Safety Data Sheet

According to 91/155 EC

Page 2/5

Printing date 31.01.2012	Reviewed on 31.01.2012

┌────────────────────────────────────────────────────────────────────────────┐

│	Trade name: Instant Adhesive	│

├────────────────────────────────────────────────────────────────────────────┤

│	(Contd. of page 1) │

│	· Treatment	│

│	In cases of irritation to the lungs, initial treatment with Dexametason │

│	metered aerosol.	│

├────────────────────────────────────────────────────────────────────────────┤

│	5 Fire-fighting measures	│

│	│

│	· Suitable extinguishing agents:	│

│	CO2, powder or water spray. Fight larger fires with water spray or	│

│	alcohol resistant foam.	│

│	· Special hazards caused by the substance, its products of combustion or	│

│	resulting gases:	│

│	Formation of toxic gases is possible during heating or in case of fire. │

│	· Protective equipment:	│

│	Wear self-contained respiratory protective device.	│

│	Do not inhale explosion gases or combustion gases.	│

│	· Additional information	│

| │ │ │ |  | Cool endangered Dispose of fire accordance with | receptacles with water spray. debris and contaminated fire fighting official regulations. |  | water in | │ │ │ |
| --- | --- | --- | --- | --- | --- | --- |
| ├────────────────────────────────────────────────────────────────────────────┤ |  |  |  |  |  |  |
| │ | 6 | Accidental release measures |  |  |  | │ |
| │ |  |  |  |  |  | │ |
| │ | · | Person-related safety precautions: |  |  |  | │ |
| │ |  | Ensure adequate ventilation |  |  |  | │ |
| │ |  | Avoid contact with the eyes and skin. |  |  |  | │ |
| │ | · | Measures for environmental protection: |  |  |  | │ |
| │ |  | Do not allow product to reach sewage system |  | or any water course. |  | │ |

│	Inform respective authorities in case of seepage into water course or	│

│	sewage system.	│

| │ | · | Measures for cleaning/collecting: |  | │ |
| --- | --- | --- | --- | --- |
| │ |  | Pick up mechanically. |  | │ |
| │ |  | Dispose of the material collected | according to regulations. | │ |
| ├────────────────────────────────────────────────────────────────────────────┤ |  |  |  |  |
| │ | 7 | Handling and storage |  | │ |
| │ |  |  |  | │ |

│	· Handling:	│

│	· Information for safe handling:	│

│	Ensure good interior ventilation, especially at floor level. (Fumes are │

│	heavier than air).	│

│	Keep receptacles tightly sealed.	│

│	· Information about fire - and explosion protection:	│

│	Keep ignition sources away - Do not smoke.	│

│	│

│	· Storage:	│

│	· Requirements to be met by storerooms and receptacles:	│

│	Store only in the original receptacle.	│

│	· Information about storage in one common storage facility: Not required. │

│	· Further information about storage conditions:	│

│	Keep receptacle tightly sealed.	│

│	Store in a cool and dry place.	│

│	Protect from heat and direct sunlight.	│

│	Store under lock and key and out of the reach of children.	│

└────────────────────────────────────────────────────────────────────────────┘

Safety Data Sheet for Instant Adhesive

Material Safety Data Sheet

According to 91/155 EC

Page 3/5

Printing date 31.01.2012	Reviewed on 31.01.2012

┌────────────────────────────────────────────────────────────────────────────┐

│	Trade name: Instant Adhesive	│

├────────────────────────────────────────────────────────────────────────────┤

│	8 Exposure controls/personal protection	│

│	│

│	· Additional information about design of technical facilities:	│

│	No further data; see item 7.	│

│	│

│	Ingredients with limit values that require monitoring at the workplace: │

│	7085-85-0 ethyl 2-cyanoacrylate	│

│	MAK (Germany): vgl. Abschn. II b	│

│	· Additional information:	│

│	The lists valid during the making were used as basis.	│

│	Ensure good ventilation/exhaustion at the workplace.	│

│	Relative humidity should be at least 50-60%.	│

│	│

│	· Personal protective equipment:	│

│	· General protective and hygienic measures:	│

│	The usual precautionary measures are to be adhered to when handling	│

│	chemicals.	│

│	Keep away from foodstuffs, beverages and feed.	│

│	Immediately remove all soiled and contaminated clothing	│

│	Wash hands before breaks and at the end of work.	│

│	Do not inhale gases / fumes / aerosols.	│

│	Avoid contact with the eyes and skin.	│

│	· Respiratory protection:	│

│	Use suitable respiratory protective device in case of insufficient	│

│	ventilation.	│

│	· Protection of hands: Protective gloves	│

│	· Eye protection: Safety glasses	│

├────────────────────────────────────────────────────────────────────────────┤

│	9 Physical and chemical properties	│

│	│

│	· Form: Fluid	│

│	· Colour: Colourless	│

│	· Odour: Irritant	│

│	·	Value/Range	Unit	Method	│

│	· Change in condition	│

│	· Melting point/Melting range:	Undetermined.	│

│	· Boiling point/Boiling range:	> 190 ° C	│

│	│

│	· Flash point:	87 ° C	│

│	│

│	· Self-igniting:	Product is not selfigniting.	│

│	│

│	· Danger of explosion:	│

│	Product does not present an	│

│	explosion hazard.	│

│	│

│	· Density:	at	20 ° C	1,06 g/cmⁿ	│

│	│

│	· Solubility in / Miscibility with	│

│	· water:	Hydrolized.	│

│	│

│	· Viscosity:	│

│	· Dynamic:	at	20 ° C	80-110 mPas	│

└────────────────────────────────────────────────────────────────────────────┘

Safety Data Sheet for Instant Adhesive

Material Safety Data Sheet

According to 91/155 EC

Page 4/5

Printing date 31.01.2012	Reviewed on 31.01.2012

┌────────────────────────────────────────────────────────────────────────────┐

│	Trade name: Instant Adhesive	│

├────────────────────────────────────────────────────────────────────────────┤

│ 10 Stability and reactivity	│

│	│

│	· Thermal decomposition / conditions to be avoided:	│

│	Protect from heat and direct sunlight.	│

│	· Dangerous reactions	│

│	Exothermic polymerization.	│

│	Reacts with alcohols, amines, aqueous acids and alkalis.	│

│	· Dangerous decomposition products:	│

│	No dangerous products of decomposition if used and stored according to	│

│	specifications.	│

├────────────────────────────────────────────────────────────────────────────┤

│ 11 Toxicological information	│

│	│

│	· Acute toxicity:	│

│	· Primary irritant effect:	│

│	· on the skin: Irritant to skin and mucous membranes.	│

│	· on the eye: Irritating effect.	│

│	· Sensitization:	│

│	Mecrilate is endangered to be sensitizing by inhalation, this cannot be │

│	excluded for ethyl 2-cyanoacrylate.	│

│	· Additional toxicological information:	│

│	The product shows the following dangers according to the calculation	│

│	method of the General EU Classification Guidelines for Preparations as	│

│	issued in the latest version:	│

│	Irritant.	│

│	No experimentally found toxicological data are available for this	│

│	preparation.	│

├────────────────────────────────────────────────────────────────────────────┤

│ 12 Ecological information	│

│	│

│	· General notes:	│

│	Water hazard class 1 (German Regulation) (Self-assessment): slightly	│

│	hazardous for water	│

│	Do not allow undiluted product or large quantities of it to reach	│

│	ground water, water course or sewage system.	│

├────────────────────────────────────────────────────────────────────────────┤

│ 13 Disposal considerations	│

│	│

│	· Product:	│

│	· Recommendation Disposal must be made according to official regulations. │

│	· European waste catalogue	│

│	08 00 00: Wastes from the manufacture, formulation, supply and use	│

│	(MFSU) of coatings, (paints, varnishes and vitreous enamels), adhesive, │

│	sealants and printing inks	│

│	08 04 00: wastes from MFSU of adhesives and sealants (including	│

│	waterproofing products)	│

│	08 04 06: adhesives and sealants sludges free of halogenated solvents	│

│	│

│	· Uncleaned packaging:	│

│	· Recommendation:	│

│	Disposal must be made according to official regulations.	│

└────────────────────────────────────────────────────────────────────────────┘

Safety Data Sheet for Instant Adhesive

Material Safety Data Sheet

According to 91/155 EC

Page 5/5

Printing date 31.01.2012	Reviewed on 31.01.2012

┌────────────────────────────────────────────────────────────────────────────┐

│	Trade name: Instant Adhesive	│

├────────────────────────────────────────────────────────────────────────────┤

│ 14 Transport information	│

│	│

│	· Transport/Additional information: Not subject to transport regulations. │

├────────────────────────────────────────────────────────────────────────────┤

│ 15 Regulatory information	│

│	│

│	· Labelling according to EU guidelines:	│

│	The product has been classified and marked in accordance with EU	│

│	Directives / Ordinance on Hazardous Materials.	│

│	│

│	· Code letter and hazard designation of product: Xi Irritant	│

│	│

│	· Risk phrases: 36/37/38 Irritating to eyes, respiratory system and skin. │

│	│

│	· Safety phrases:	│

│	2	Keep out of the reach of children.	│

│	23	Do not breathe fumes.	│

│	24/25	Avoid contact with skin and eyes.	│

│	26	In case of contact with eyes, rinse immediately with plenty of │

│	water and seek medical advice.	│

│	│

│	· Special labelling of certain preparations:	│

│	Cyanacrylate - DANGER! Glues skin and eyelids within seconds. Should be │

│	kept out of children's reach.	│

│	│

│	· National regulations:	│

│	│

│	· Water hazard class:	│

│	Water hazard class 1 (Self-assessment): slightly hazardous for water.	│

├────────────────────────────────────────────────────────────────────────────┤

│ 16 Other information	│

│	│

│	This information is based on our present knowledge. However, this shall │

│	not constitute a guarantee for any specific product features and shall	│

│	not establish a legally valid contractual relationship.	│

└────────────────────────────────────────────────────────────────────────────┘