MSDS#: NKM90803-DDM

Material Safety Data Sheet	DIAMOND D SELF CURE MONOMER	Page 1 of 6

Section I - Product and Company Identification

Product Name:	DIAMOND D SELF CURE MONOMER	MSDS#:	NKM090803-DDM

Chemical Name:	Methyl Methacrylate	MSDS Approval

Date:

9/10/2003	MSDS Prepared by:

BSQ

Family: Acrylic Monomers	Manufacturer:  Keystone Industries

616 Hollywood Ave, Cherry Hill, NJ 08002

Product Use: Dental Monomer	Emergency Phone Numbers: 131126

Formula: Proprietary Formulation	Information Contacts: (03) 95321799

Section II – Composition/Information on Ingredients

Chemical Identity		CAS Numbers

EINECS#	INCI Name	Exposure	 Limits	Carcinogen	% OSHA	ACGIH

TWA/STEL	TWA/STEL	IARC/NTP/OSHA

Methyl Methacrylate	80-62-6	201-297-1	Methyl methacrylate	100 ppm	50 ppm/100 ppm	Group 3/no/no	>90

Ethylene Glycol Dimethacrylate	97-90-5	202-617-2	N/DA	N/E	N/E	Not Listed	<10

Inhibitor (MEHQ)	150-76-5	205-769-8	p-Hydroxyanisole	N/E	5 mg/m3	Not Listed	<1

N/E - None Established

N/R - Not Reviewed

Hazard Symbols: Xi F

N/DA - No Data Available

N/A - Not Applicable

Risk Phrases: R11, R36/37/38, R43

Safety Phrases: S9, S16, S29, S33, S36/37/39, S45

Section III - Hazards Identification 		 EMERGENCY OVERVIEW

This information is based on findings from related or similar materials.

|  | Danger! Flammable liquid and vapor. |  | May cause allergic skin reaction. |
| --- | --- | --- | --- |
|  | Known Sensitizer. |  | Light and Air sensitive. |
|  | May cause eye irritation. |  | Target Organs: Kidneys, central nervous system, liver. |
|  | May cause respiratory tract irritation. |  |  |

Potential Health Effects, Signs and Symptoms of Exposure:

Primary Route of Entry	Inhalation , skin , eyes

Eye	Vapor concentrations may cause irritation of eyes. Liquid contact with eyes can cause irritation and possible corneal damage.

Skin	Liquid concentration may cause severe skin irritation. Repeated or prolonged contact may cause allergic skin rashes, itching and swelling which becomes evident on re-exposure to this product.

Ingestion	May cause central nervous system depression, kidney damage, and liver damage. May cause irritation, a burning sensation of the mouth, throat, respiratory tract, and abdominal pain.

Inhalation	High vapor concentrations may irritate the respiratory system. Prolonged exposure can lead to headaches, nausea, drowsiness, unconsciousness, and coma.

Sub-Chronic Effects	Prolonged or repated skin contact may cause sensitization dermatitis and possible destruction and/or ulceration. May cause reproductive and fetal effects. Repeated exposure may cause tingling in the extremitites and other nervous system abnormalities.

NOTE: Refer to Section 11, Toxicological Information for Details

Reviewed Date: 3/2/2012

MSDS#: NKM90803-DDM

Material Safety Data Sheet	DIAMOND D SELF CURE MONOMER	Page 2 of 6

Section IV - First Aid Measures

First Aid for Eye	Immediately flush eyes with water for 15 minutes, occasionally lifting the upper and lower eyelids. Get medical help if discomfort persists.

First Aid for Skin	Wash thoroughly with soap and water. Remove contaminated clothing. Get medical help if discomfort persists.Wash clothing before use.

First Aid for Inhalation	Remove to fresh air. If having breathing difficulty, give oxygen.  If breathing has stopped, give artificial respiration. Get medical help if discomfort persists.

First Aid for Ingestion	Never give anything by mouth to an unconscious person. Get medial aid. Do NOT induce vomiting. If conscious and alert, rinse mouth and drink 2 to 4 cupfuls of milk or water.

Section V - Fire Fighting Measures

| Flash Point (°F/°C) | Flammable Limit (vol%) | Auto-ignition Temperature (vol%) |
| --- | --- | --- |
| Tag Closed Cup: 68F/20ºC | LEL: 2.0% UEL: 12.5% | 790ºF/421ºC |

Method:

Extinguishing Media:	Foam, Carbon Dioxide, Dry Chemical.

Fire Fighting

Instructions:

Wear self-contained breathing apparatus and full protective gear. Water may be ineffective unless used as a fine spray or fog. Use water spray to cool the exposed containers of methacrylate monomer.

Unusual Hazards:	Vapors  may  travel to  source  of  ignition and  flash  back. Avoid  ignition  sources  or  excessive temperatures .Heat  can  induce  polymerization  with rapid  release  of  energy.  Closed  containers may  rupture explosively. Spontaneous polymerization  may  occur on prolonged aging.

Section VI - Accidental Release Measures

Spill or Release

Procedures

Eliminate all sources of heat and ignition. Use absorbent material for spills and dike it, wash spill material into retaining containers.  Place containers in a well ventilated area. Consult  an  expert  on disposal of  recovered  material  and  ensure conformity  to  local disposal  regulations. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Use non-sparking tools and equipment. Collect liquid in an appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as sawdust. Do not flush to sewer! US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802. If a leak or spill has not

ignited, use water spray to disperse the vapors, to protect personnel attempting to stop leak, and to flush spills away from exposures.

Section VII - Handling and Storage

Handling	Keep away from heat, sparks, flames and other sources of ignition. Avoid contact with eyes, skin and clothing. Avoid breathing vapor or mist. Use with adequate ventilation. Ground all metal containers when transferring and use explosion-proof equipment. Follow all MSDS/label precautions even after the container is emptied because it may retain product residues. Wash thoroughly after handling.

Storage	Store in a cool,  dry  area.  Keep  container  closed when  not  in use. Store  at  ambient temperatures out  of  direct sunlight. Store in a well ventilated place. Store in accordance with National Fire Protection Association recommendations. Maintain air space inside storage containers. Inhibitor requires air (oxygen) contact to function.  Check inhibitor levels after 3 months and return to original level.

Explosion Hazard	Avoid  ignition  sources or  excessive temperatures. Heat can  induce polymerization  with  rapid release of energy. Closed containers may rupture explosively. Spontaneous polymerization may occur on prolonged aging.

Reviewed Date: 3/2/2012

MSDS#: NKM90803-DDM

Material Safety Data Sheet	DIAMOND D SELF CURE MONOMER	Page 3 of 6

Section VIII - Exposure Controls / Personal Protective Equipment

Engineering Controls	Facilities storing or ultilizing this material should be equipped with an eye facility and safety shower.

Use process enclosures local exhaust ventilation, or other engineering controls to control airborne levels below recommended exposure limits. Use explosion-proof ventilation equipment.

Methyl methacrylate: IDLH = 1000 ppm via NIOSH standards.

Personal Protective Equipment

General	To  identify  additional  Personal Protective  Equipment  (PPE) requirements,  it  is recommended that

a  hazard  assessment  in  accordance with the OSHA  PPE  Standard (29CFR1910.132), or European Standard EN166  be  conducted  before  using  this product . Provide  eye  wash  stations and  safety showers. Wear  impervious  clothing  to  prevent  ANY  contact  with  this  product,  such  as  gloves, apron,  boots, or  whole body  suit. Nitrile rubber  is better  than  PVC.

Eye/ Face Protection	Wear safety glasses.  Wear coverall chemical splash goggles and face shield when possibility exists for eye and face contact due to splashing or spraying material.

Skin Protection	Use impermeable clothing to prevent ANY contact with this product, such as gloves, apron, boots, or whole body suit. Nitrile rubber is better than PVC.

Respiratory Protection	A NIOSH/MSHA approved air purifying respirator with an organic vapor cartridge or canister may be permissible under certain limited circumstances where airborne concentrations are expected to exceed exsposure limits. Protection provided by air purifying respirators is limited. Wear a NIOSH/MSHA or European Standard EN 149 approved full-facepeice airline respirator in the positive pressure mode with emergency escape provisions. Follow OSHA repsirator regulations found in 29 CFR 1910.134 or Eurpean Standard EN 149.

Section IX - Physical and Chemical Properties

| Appearance |  | Odor & Odor Threshold |  |  | PH |  | Specific Gravity |  | Viscosity |  | % Volatile |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clear, colorless liquid |  | Characteristic strong, acrid odor |  |  | N/A |  | (H20=1): 0.94 |  | N/DA, mPas @ 20ºC |  | W/W %: 99+ |  |
| Boiling Point/ Freezing Point | Decomposition Temperature |  | Octanol/Water Partitioning Coefficient Log Po/w | Vapor Pressure: |  | Vapor Density |  | Evaporation Rate |  | Ignition |  | Solubility In Water (20°C) |
| 214ºF/101ºC N/DA | N/A |  | N/DA | mm Hg : 29 @ 25ºC |  | (Air =1): 3.45 |  | (Butyl Acetate = 1): 1.45 |  | N/DA |  | Slightly soluble |

| Flash Point (°F/°C) | Flammable Limit (vol%) | Auto-ignition Temperature (vol%) |
| --- | --- | --- |
| Tag Closed Cup: 68F/20ºC | LEL: 2.0% UEL: 12.5% | 790ºF/421ºC |

Section X - Stability and Reactivity

Stability:	Incompatibility (Materials to Avoid): Stable	Reducing and oxidizing agents and UV light. Hazardous Decomposition Products:	Hazardous Polymerization:

Oxides of carbon when burned.	May occur

Conditions to Avoid:

Temperatures above 40ºC, oxidizing or reducing agents, peroxides and amines, storage in absence of inhibitor, and inadvertent addition of

catalyst. Avoid aging and contamination.

Section XI - Toxicological Information

| Acute Oral Toxicity | Acute Dermal Toxicity | Acute Inhalation Toxicity | Irritation - skin | Irritation - Eye |
| --- | --- | --- | --- | --- |
| Oral(Rat) LD50: 7872 mg/kg | Dermal (Rabbit) LD50: 9400mg/kg | Inhalation (Rat) LC50 3750ppm | N/DA | N/DA |

| Sensitization | Mutagenicity | Sub-chronic Toxicity |
| --- | --- | --- |
| N/DA | N/DA | N/DA |

Reviewed Date: 3/2/2012

MSDS#: NKM90803-DDM

Material Safety Data Sheet	DIAMOND D SELF CURE MONOMER	Page 4 of 6

Section XII - Ecological Information

Ecotoxicological Information

| Acute Toxicity to Fish | Acute Toxicity to Invertebrates | Acute Toxicity to Algae | Bioconcentration | Toxicity to Sewage Bacteria |
| --- | --- | --- | --- | --- |
| 96 hour LC50: fathead minnows: 150 ppm bluegill sunfish; 232 ppm | N/DA | N/DA | N/DA | N/DA |

Chemical Fate Information

| Biodegradability | N/DA |
| --- | --- |
| Chemical Oxygen Demand | N/DA |

Section XIII - Disposable Considerations

Dispose of diking materials and absorbent in compliance with State, Local, and Federal regulations.  Residual vapors may explode on  ignition; do  not cut, drill, or weld  on  or near  the  container.  Mix with  compatible  chemical  which is less  flammable  and  incinerate.

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance

with federal, state and local requirements.

Section XIV - Transport Information

| DOT (49 CFR 172) |  |
| --- | --- |
| Proper Shipping Name: | Flammable liquids, n.o.s., (methyl methacrylate, ethylene glycol dimethacrylate), 3, UN1993, PGII |
| Identification Number: | UN1993 |
| Marine Pollutant: | No |
| Special Provisions: | T8, T31 |
| Emergency Response Guidebook (ERG) #: | 128 |
| IATA (DGR): |  |
| Proper Shipping Name: | Flammable liquids, n.o.s., (methyl methacrylate, ethylene glycol dimethacrylate), 3, UN1993, PGII |
| Class or Division: | 3 |
| UN or ID Number: | UN1993 |
| Packaging Instructions: |  |
| Emergency Response Guidance (ICAO)#: | 3L |
| IMO (IMDG): |  |
| Proper Shipping Name: | Flammable liquids, n.o.s., (methyl methacrylate, ethylene glycol dimethacrylate), 3, UN1993, PGII |
| Class or Division: | 3.2 |
| UN or ID Number: | UN1993 |
| Special Provisions & Stowage/Segregation: | None |
| Emergency Schedule (EmS)#: |  |
| Other Information: | Flash point = 20ºC |

Section XV - Regulatory Information

US Federal Regulations

Clean Air Act: HAP/ODS	This product contains the following hazardous air pollutants (HAP) as defined by the U.S.

Clean Air Act:

	Methyl methacrylate, CAS# 80-62-6

This product contains no Class 1 or Class 2 ODS (Ozone Depleting Substance).

Reviewed Date: 3/2/2012

MSDS#: NKM90803-DDM

Material Safety Data Sheet	DIAMOND D SELF CURE MONOMER	Page 5 of 6

| Clean Water Act: Priority Pollutant/Hazardous Substance | This product contains the following Hazardous Substances as defined by the CWA:  Methyl methacrylate, CAS# 80-62-6 This product does not contain any substances that are a Priority Pollutant or Toxic Pollutant under the CWA. |  |
| --- | --- | --- |
| FDA: Food Packaging Status | This product has not been cleared by the FDA for use in food packaging and/or other applications as an indirect food additive. |  |
| Occupational Safety and Health Act | This product is considered to be a hazardous chemical under the OSHA Hazard Communication Standard. Its hazards are:  Immediate (acute) health hazard  Fire hazard  Reactive hazard |  |
| RCRA | This product contains chemicals considered to be hazardous waste under RCRA (40 CFR 261):  Methyl methacrylate CAS# 80-62-6, RCRA Code U162  Characteristic of Ignitablility: RCRA Code: D001 |  |
| SARA Title III: Section 302 (TPQ) | This product contains no chemicals regulated under Sec. 302 as extremely hazardous substances that carry a TPQ. |  |
| SARA Title III: Section 302 (RQ) | This product contains chemicals regulated under Section 302 as extremely hazardous chemicals for emergency release notification ("CERCLA" List):  Methyl methacrylate CAS# 80-62-6, RQ(Lbs): 1000 |  |
| SARA Title III: Section 311- 312: | This product is considered hazardous under the OSHA Hazard Communication Standard and is regulated under Section 311-312 (40 CFR 370). Its hazards are:  Immediate (acute) health  Fire hazard  Delayed (chronic) health  Reactive hazard |  |
| SARA Title III: Section 313: | This product contains the following substances subject to the reporting requirements of Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR Part 372:  Methyl methacrylate, CAS# 80-62-6 |  |
| TSCA Section 8(b): Inventory: | This product contains chemicals that are on the TSCA list. |  |

State Regulations

| CA Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, 4-Methoxyphenol, CAS# 150-76-5 California No Significant Risk Level: None of the chemicals in this product are listed. |
| --- | --- |
| MA Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, 4-Methoxyphenol, CAS# 150-76-5 |
| NJ Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, 4-Methoxyphenol, CAS# 150-76-5 |
| PA Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, 4-Methoxyphenol, CAS# 150-76-5 |
| FL Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, 4-Methoxyphenol, CAS# 150-76-5 |
| MN Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, 4-Methoxyphenol, CAS# 150-76-5 |

Reviewed Date: 3/2/2012

MSDS#: NKM90803-DDM

Material Safety Data Sheet	DIAMOND D SELF CURE MONOMER	Page 6 of 6

International Regulations

| CDSL: Canadian Inventory (on Canadian Transitional List) | Methyl methacrylate, CAS# 80-62-6 is on the DSL List. WHMIS = B2, D2B. Ethylene glycol dimethacrylate, CAS# 97-90-5 is on the DSL List. WHMIS = n/da 4-Methoxyphenol, CAS# 150-76-5 is on the DSL List. WHMIS = n/da |
| --- | --- |
| EINECS: European Inventory: | Diamond D Self Cure Monomer:  HAZARD SYMBOLS: Xi, F: Irritant, Highly Flammable  RISK PHRASES: R11: highly flammable, R36/37/38: Irritating to eyes, respiratory system and skin, R43: May cause sensitization by skin contact  SAFETY PHRASES: S9: keep container in a well ventilated place, S16: keep away from sources of ignition- no smoking, S29: do not empty into drains, S33: take precautionary measures against static discharges, S36/37/39: wear suitable protective clothing, gloves and eye/face protection, S45: In case of accident or if you feel unwell, seek medical advise immediately (show the label where possible) |

Section XVI - Other Information

Hazard Rating System (Pictograms)

NFPA:	HMIS:

2

3

2

Flammability

Health

3

2	2

Reactivity

| Product Number - |  |
| --- | --- |
| Revised Sections since Last Version: |  |
| 9/10/2003 | Approval date |
| 2/15/2006 | Reviewed. No changes made |
| 1/25/2010 | Reviewed. No changes made |
| 3/30/2011 | Added Methyl Mecrylate as Chemical name in section 1 |

The information presented herein was obtained from sources considered to be reliable.  However, this information is provided without any warranty, expressed or implied, regarding its correctness or suitability for consumers intended use and/or application.  For this and other reasons, we assume no responsibility and expressly disclaim liability for loss, damage or expense arising out of any way connected with the handling, storage, use or disposal of the product.  This MSDS was prepared expressly for this product.  Use the materials only as directed.  If the product is used as a component of another product, the information contained within the MSDS may not be applicable.  If one could have any concerns with or problems understanding this MSDS form, please direct all questions to INFOTRAC, Chemical Emergency Resources System at 131126.

Reviewed Date: 3/2/2012