Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

HEAT CURE MONOMER	Page 1 of 7

| Chemical Name: Plasticized methacrylate monomer Section I - Product and Company Identification |  |  |  |
| --- | --- | --- | --- |
| Product Name: VERSACRYL PREMIX HEAT CURE MONOMER |  | MSDS#: KIM071103-VRH |  |
|  | MSDS Approval MSDS Prepared Date: 2/2/2012 by: BSQ |  |  |

Family: Acrylic Monomers	Manufacturer:  KEYSTONE INDUSTRIES

616 Hollywood Ave, Cherry Hill, NJ 08002

Product Use: Organic Process Chemical	Emergency Phone Numbers: 13 11 26

Product #: 1014004, 1014014, 1014019	Information Contacts: 03 9532 1799

Section II – Composition/Information on Ingredients

Chemical Identity		CAS Numbers

EINECS#	INCI Name	Exposure		Limits	Carcinogen	% OSHA	ACGIH

TWA/STEL	TWA/STEL	IARC/NTP/OSHA

Methyl Methacrylate	80-62-6	201-297-1	N/DA	100 ppm	50 ppm/100 ppm	Group 3/no/no	<40

Dibutyl Phthalate	84-74-2	201-557-4	Dibutyl phthalate	5 mg/m3	N/E	Not Listed	<30

Dioctyl Phthalate	117-81-7	204-211-0	Diethylhexyl phthalate	5 mg/m3	5 mg/m3	Group 3/Suspect/ Possible Select

<25

n-Butyl Methacrylate	97-88-1	202-615-1	Butyl Methacrylate	N/E	N/E	Not listed	<5

Ethylene Glycol Dimethacrylate	97-90-5	202-617-2	N/DA	N/E	N/E	Not Listed	<5

N/E - None Established

N/R - Not Reviewed

Hazard Symbols: Xn F

N/DA - No Data Available

N/A - Not Applicable

Risk Phrases: R11, R36/37/38, R40, R43

Safety Phrases:  S2, S9, S16, S24/25, S29, S36/37/39, S45

Section III - Hazards Identification 		 EMERGENCY OVERVIEW

This information is based on findings from related or similar materials.

•	Danger! Flammable liquid and vapor.	•	May cause allergic skin reaction.

•	Known Sensitizer.	•	Light and Air sensitive.

•	May cause eye irritation.	•	Target Organs: Kidneys, central nervous system, liver.

•	May cause respiratory tract irritation.	•	Possible cancer hazard, read MSDS for further details.

| Potential Health Effects, Signs and Symptoms of Exposure: Primary Route of Entry | Inhalation , skin , eyes |
| --- | --- |
| Eye | Vapor concentrations may cause irritation of eyes. Liquid contact with eyes can cause irritation and possible corneal damage. |
| Skin | Liquid concentration may cause severe skin irritation. Repeated or prolonged contact may cause allergic skin rashes, itching and swelling which becomes evident on re-exposure to this product. |
| Ingestion | May cause central nervous system depression, kidney damage, and liver damage. May cause irritation, a burning sensation of the mouth, throat, respiratory tract, and abdominal pain. |
| Inhalation | High vapor concentrations may irritate the respiratory system. Prolonged exposure can lead to headaches, nausea, drowsiness, unconsciousness, and coma. |
| Sub-Chronic Effects (mixture) | Prolonged or repated skin contact may cause sensitization dermatitis and possible destruction and/or ulceration. May cause reproductive and fetal effects. Repeated exposure may cause tingling in the extremitites and other nervous system abnormalities. |
| Chronic Effects, Dioctyl Phthalate (DEHP) | DEHP toxicity appears to be a high-dose phenomenon readily demonstrable in some, but not all rodent species and strains. Liver toxicity, so characteristic of rodent responses to DEHP, appears to be irrelevant to humans. The carcinogenic response of DEHP has been demonstrated only in one strain of rat and mouse and does not appear to be a feature of toxicity in higher order mammals, especially humans. Reproductive and developmental toxicity, likewise, appears to be limited to high-dose effects seen in rodent testing. The relevance to human testing has not been established. |

NOTE: Refer to Section 11, Toxicological Information for Details

Revised Date: 2/2/12 | Replaces Date: 2/4/04

Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

HEAT CURE MONOMER	Page 2 of 7

Section IV - First Aid Measures

| First Aid for Eye | If symptoms develop, move individual away from exposure and into fresh air. Flush eyes gently with water for at least 15 min. while holding eyelids apart. If symptoms persist or there is any visual difficulty, seek medical attention. |
| --- | --- |
| First Aid for Skin | Wash thoroughly with soap and water. Remove contaminated clothing. Get medical help if discomfort persists.Wash clothing before use. |
| First Aid for Inhalation | Remove to fresh air. If having breathing difficulty, give oxygen. If breathing has stopped, give artificial respiration. Get medical help if discomfort persists. |
| First Aid for Ingestion | Never give anything by mouth to an unconscious person. Get medial aid. Do NOT induce vomiting. If conscious and alert, rinse mouth and drink 2 to 4 cupfuls of milk or water. |

|  | Section V - Fire Fighting Measures |  |  |  |
| --- | --- | --- | --- | --- |
| Flash Point (°F/°C) |  | Flammable Limit (vol%) | Auto-ignition Temperature (vol%) |  |
| Tag Closed Cup: 51°F/10ºC |  | LEL: 2.12% UEL: 12.5% | 815ºF/435ºC |  |

| Method: |  |
| --- | --- |
| Extinguishing Media: | Foam, Carbon Dioxide, Dry Chemical. |
| Fire Fighting Instructions: | Wear self-contained breathing apparatus and full protective gear. Water may be ineffective unless used as a fine spray or fog. Use water spray to cool the exposed containers of methacrylate monomer. |
| Unusual Hazards: | Vapors may travel to source of ignition and flash back. Avoid ignition sources or excessive temperatures .Heat can induce polymerization with rapid release of energy. Closed containers may rupture explosively. Spontaneous polymerization may occur on prolonged aging. |

Section VI - Accidental Release Measures

Spill or Release

Procedures

Eliminate all sources of heat and ignition. Use absorbent material for spills and dike it, wash spill material into retaining containers.   Place containers in a well ventilated area. Consult  an  expert  on disposal  of  recovered  material  and  ensure  conformity  to  local  disposal  regulations. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Use non-sparking tools and equipment. Collect liquid in an appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as sawdust. Do not flush to sewer! US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802. If a leak or spill has not

ignited, use water spray to disperse the vapors, to protect personnel attempting to stop leak, and to flush spills away from exposures.

| Section VII - Handling and Storage Handling | Keep away from heat, sparks, flames and other sources of ignition. Avoid contact with eyes, skin and clothing. Avoid breathing vapor or mist. Use with adequate ventilation. Ground all metal containers when transferring and use explosion-proof equipment. Follow all MSDS/label precautions even after the container is emptied because it may retain product residues. Wash thoroughly after handling. |
| --- | --- |
| Storage | Store in a cool, dry area. Keep container closed when not in use. Store at ambient temperatures out of direct sunlight. Store in a well ventilated place. Store in accordance with National Fire Protection Association recommendations. Maintain air space inside storage containers. Inhibitor requires air (oxygen) contact to function. Check inhibitor levels after 3 months and return to original level. |
| Explosion Hazard | Avoid ignition sources or excessive temperatures. Heat can induce polymerization with rapid release of energy. Closed containers may rupture explosively. Spontaneous polymerization may occur on prolonged aging. |

Revised Date: 2/2/12 | Replaces Date: 2/4/04

Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

HEAT CURE MONOMER	Page 3 of 7

Section VIII - Exposure Controls / Personal Protective Equipment

Engineering Controls	Facilities storing or ultilizing this material should be equipped with an eye facility and safety shower.

Use process enclosures local exhaust ventilation, or other engineering controls to control airborne levels below recommended exposure limits. Use explosion-proof ventilation equipment.

Methyl methacrylate: IDLH = 1000 ppm via NIOSH standards. Dibutyl Phthalate: IDLH = 4000 mg/m3  via NIOSH standards.

| Personal Protective Equipment General | To identify additional Personal Protective Equipment (PPE) requirements, it is recommended that a hazard assessment in accordance with the OSHA PPE Standard (29CFR1910.132), or European Standard EN166 be conducted before using this product . Provide eye wash stations and safety showers. Wear impervious clothing to prevent ANY contact with this product, such as gloves, apron, boots, or whole body suit. Nitrile rubber is better than PVC. |
| --- | --- |
| Eye/ Face Protection | Wear safety glasses. Wear coverall chemical splash goggles and face shield when possibility exists for eye and face contact due to splashing or spraying material. |
| Skin Protection | Use impermeable clothing to prevent ANY contact with this product, such as gloves, apron, boots, or whole body suit. Nitrile rubber is better than PVC. |
| Respiratory Protection | A NIOSH/MSHA approved air purifying respirator with an organic vapor cartridge or canister may be permissible under certain limited circumstances where airborne concentrations are expected to exceed exsposure limits. Protection provided by air purifying respirators is limited. Wear a NIOSH/MSHA or European Standard EN 149 approved full-facepeice airline respirator in the positive pressure mode with emergency escape provisions. Follow OSHA repsirator regulations found in 29 CFR 1910.134 or European Standard EN 149. |

Section IX - Physical and Chemical Properties

| Appearance |  | Odor & Odor Threshold |  |  | PH | Specific Gravity |  | Viscosity |  | % Volatile |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clear, pale blue liquid |  | Acrid, fruity OT = N/D |  |  | N/D | (H20=1): 0.94 |  | Like water |  | W/W %: 99+ |  |
| Boiling Point/ Freezing Point | Decomposition Temperature |  | Octanol/Water Partitioning Coefficient Log Po/w | Vapor Pressure: |  | Vapor Density | Evaporation Rate |  | Ignition |  | Solubility In Water (20°C) |
| 214ºF/101ºC N/DA | N/A |  | N/DA | mm Hg : 29 @ 20ºC |  | (Air =1): 3.5 | (Butyl Acetate= 1): 3.0 |  | N/DA |  | Moderate, 1.6gm/100gm @20ºC |

| Flash Point (°F/°C) | Flammable Limit (vol%) | Auto-ignition Temperature (vol%) |
| --- | --- | --- |
| Tag Closed Cup: 68°F/20ºC | LEL: 2.0% UEL: 12.5% | 790ºF/421ºC |

Section X - Stability and Reactivity

Stability:	Incompatibility (Materials to Avoid): Stable	Reducing and oxidizing agents and UV light. Hazardous Decomposition Products:	Hazardous Polymerization:

Oxides of carbon when burned.	May occur

Conditions to Avoid:

Temperatures above 40ºC, oxidizing or reducing agents, peroxides and amines, storage in absence of inhibitor, and  inadvertent

addition of catalyst. Avoid aging and contamination.

Revised Date: 2/2/12 | Replaces Date: 2/4/04

Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

HEAT CURE MONOMER	Page 4 of 7

Section XI - Toxicological Information

| Acute Oral Toxicity | Acute Dermal Toxicity | Acute Inhalation Toxicity | Irritation - skin | Irritation - Eye |
| --- | --- | --- | --- | --- |
| Oral(Rat) LD50: 2297gm/kg (mixture) | Dermal (rabbit) LD50: 8125mg/kg (mixture) | Inhalation (Rat) LC50 >12,500 to 16,500ppm for 0.5 hours (MMA) | N/DA | Slight |

| Sensitization | Mutagenicity | Sub-chronic Toxicity |
| --- | --- | --- |
| N/DA | Hamster, ovary, fibroblast, oral(cytogenetic analysis): 887 mg/l (mixture) | N/DA |

Section XII - Ecological Information

Ecotoxicological Information

| Acute Toxicity to Fish | Acute Toxicity to Invertebrates | Acute Toxicity to Algae | Bioconcentration | Toxicity to Sewage Bacteria |
| --- | --- | --- | --- | --- |
| 96 hour LC50 (mixture): fathead minnows:100-1000 ppm goldfish: 58 ppm | N/DA | N/DA | N/DA | N/DA |

Chemical Fate Information

| Biodegradability | N/DA |
| --- | --- |
| Chemical Oxygen Demand | N/DA |

Section XIII - Disposal Considerations

Dispose of diking materials and absorbent in compliance with State, Local, and Federal regulations.  Residual  vapors  may  explode  on ignition;  do  not  cut,  drill,  or  weld  on  or  near  the container.  Mix  with  compatible  chemical  which  is  less  flammable  and incinerate.

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ

from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

|  | Section XIV - Transport Information |  |  |
| --- | --- | --- | --- |
| DOT (49 CFR 172) |  |  |  |
| Proper Shipping Name: |  | Flammable liquids, n.o.s., (methyl methacrylate, dibutyl phthalate), 3, UN1993, PGII |  |
| Identification Number: |  | UN1993 |  |
| Marine Pollutant: |  | No |  |
| Special Provisions: |  | T8, T31 |  |
| Emergency Response Guidebook (ERG) #: |  | 128 |  |
| IATA (DGR): |  |  |  |
| Proper Shipping Name: |  | Flammable liquids, n.o.s., (methyl methacrylate, dibutyl phthalate), 3, UN1993, PGII |  |
| Class or Division: |  | 3 |  |
| UN or ID Number: |  | UN1993 |  |
| Packaging Instructions: |  |  |  |
| Emergency Response Guidance (ICAO)#: |  | 3L |  |
| IMO (IMDG): |  |  |  |
| Proper Shipping Name: |  | Flammable liquids, n.o.s., (methyl methacrylate, dibutyl phthalate), 3, UN1993, PGII |  |
| Class or Division: |  | 3.2 |  |
| UN or ID Number: |  | UN1993 |  |
| Special Provisions & Stowage/Segregation: |  | None |  |

Revised Date: 2/2/12 | Replaces Date: 2/4/04

Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

HEAT CURE MONOMER	Page 5 of 7

| Emergency Schedule (EmS)#: |  |
| --- | --- |
| Other Information: | Flash point = 10ºC |

Section XV - Regulatory Information

US Federal Regulations

| Clean Air Act: HAP/ODS | This product contains the following hazardous air pollutants (HAP) as defined by the U.S. Clean Air Act: • Methyl methacrylate, CAS# 80-62-6 • Dibutyl phthalate, CAS# 84-74-2 • Dioctyl phthalate, CAS# 117-81-7 This product contains no Class 1 or Class 2 ODS. |
| --- | --- |
| Clean Water Act: Priority Pollutant/Hazardous Substance | This product contains the following Hazardous Substances as defined by the CWA: • Methyl methacrylate, CAS# 80-62-6 • Dibutyl phthalate, CAS# 84-74-2 This product contains the following substances that are a Priority Pollutant: • Dioctyl phthalate, CAS# 117-81-7. This product does not contain any substances that are a Toxic Pollutant under the CWA. |
| FDA: Food Packaging Status | This product has not been cleared by the FDA for use in food packaging and/or other applications as an indirect food additive. |
| Occupational Safety and Health Act | This product is considered to be a hazardous chemical under the OSHA Hazard Communication Standard. Its hazards are: • Immediate (acute) health hazard • Fire hazard • Chronic (delayed) health hazard |
| RCRA | This product contains chemicals considered to be hazardous waste under RCRA (40 CFR 261): • Methyl methacrylate CAS# 80-62-6, RCRA Code U162 • Dibutyl phthalate, CAS# 84-74-2, RCRA Code U069 • Dioctyl phthalate, CAS# 117-81-7, RCRA Code U028 • Characteristic of Ignitablility: RCRA Code: D001 |
| SARA Title III: Section 302 (TPQ) | This product contains no chemicals regulated under Sec. 302 as extremely hazardous substances that carry a TPQ. |
| SARA Title III: Section 302 (RQ) | This product contains chemicals regulated under Section 302 as extremely hazardous chemicals for emergency release notification ("CERCLA" List): • Methyl methacrylate CAS# 80-62-6, RQ(Lbs): 1000 • Dibutyl phthalate, CAS# 84-74-2, RQ(Lbs): 10 • Dioctyl phthalate, CAS# 117-81-7, RQ(Lbs): 100 |
| SARA Title III: Section 311-312: | This product is considered hazardous under the OSHA Hazard Communication Standard and is regulated under Section 311-312 (40 CFR 370). Its hazards are: • Immediate (acute) health • Fire hazard • Delayed (chronic) health hazard |
| SARA Title III: Section 313: | This product contains the following substances subject to the reporting requirements of Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR Part 372: • Methyl methacrylate, CAS# 80-62-6 • Dibutyl phthalate, CAS# 84-74-2 • Dioctyl phthalate, CAS# 117-81-7 |

Revised Date: 2/2/12 | Replaces Date: 2/4/04

Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

TSCA Section 8(b): Inventory: TSCA Significant New Use Rule:

HEAT CURE MONOMER	Page 6 of 7

This product contains chemicals that are on the TSCA list.

None of the chemicals in this material have a SNUR under TSCA.

State Regulations

| CA Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, Dibutyl phthalate, CAS# 84-74-2, Dioctyl phthalate, CAS# 117- 81-7 California No Significant Risk Level: Dioctyl phthalate, CAS# 117-81-7 = 80 µg/day California Safe Drinking Water Act: The following statement(s) is(are) made in order to comply with the CSWDA: WARNING: This product contains Dioctyl Phthalate, a chemical known to the state of California to cause cancer. |
| --- | --- |
| MA Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, Dibutyl phthalate, CAS# 84-74-2, Dioctyl phthalate, CAS# 117- 81-7, Butyl methacrylate, CAS# 97-88-1 |
| NJ Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, Dibutyl phthalate, CAS# 84-74-2, Dioctyl phthalate, CAS# 117- 81-7, Butyl methacrylate, CAS# 97-88-1 |
| PA Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, Dibutyl phthalate, CAS# 84-74-2, Dioctyl phthalate, CAS# 117- 81-7, Butyl methacrylate, CAS# 97-88-1 |
| FL Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, Dibutyl phthalate, CAS# 84-74-2, Dioctyl phthalate, CAS# 117- 81-7, Butyl methacrylate, CAS# 97-88-1 |
| MN Right-to-Know Law: | Methyl methacrylate, CAS# 80-62-6, Dibutyl phthalate, CAS# 84-74-2, Dioctyl phthalate, CAS# 117- 81-7 |

International Regulations

| CDSL: Canadian Inventory (on Canadian Transitional List) | Methyl methacrylate, CAS# 80-62-6 is on the DSL List. WHMIS = B2, D2B. Dibutyl phthalate, CAS# 84-74-2 is on the DSL List. WHMIS = D2B Dioctyl phthalate, CAS# 117-81-7 is on the DSL List. WHMIS = D2A Butyl Methacrylate, CAS# 97-88-1 is on the DSL List. WHMIS = B2, D2A, F Ethylene glycol dimethacrylate, CAS# 97-90-5 is on the DSL List. WHMIS = n/da |
| --- | --- |
| EINECS: European Inventory: | Versacryl Reline Heat Cure Monomer: • HAZARD SYMBOLS: Xn, F: Harmful, Highly Flammable • RISK PHRASES: R11: highly flammable, R36/37/38: Irritating to eyes, respiratory system and skin, R40: Possible risks of irreversible effects, R43: May cause sensitization by skin contact • SAFETY PHRASES: S2: Keep out of the reach of children, S9: keep container in a well ventilated place, S16: keep away from sources of ignition- no smoking, S24/25: Avoid contact with skin & eyes, 29: do not empty into drains, S36/37/39: wear suitable protective clothing, gloves and eye/face protection, S45: In case of accident or if you feel unwell, seek medical advise immediately (show the label where possible) |

Section XVI - Other Information

Hazard Rating System (Pictograms)

| NFPA: HMIS: Flammability 3 2 2 Health Reactivity | 2 |  |
| --- | --- | --- |
|  | 3 |  |
|  | 2 |  |

Revised Date: 2/2/12 | Replaces Date: 2/4/04

Material Safety Data Sheet	VERSACRYL PREMIX

MSDS#: KIM071103-VRH

HEAT CURE MONOMER	Page 7 of 7

*- Gloves and Safety Glasses or Chemical Splash Goggles

Revised Sections since Last Version:	Heading, MSDS name changed from Reline to Premix.  Product numbers added.

The information presented herein was obtained from sources considered to be reliable.  However, this information is provided without any warranty, expressed or implied, regarding its correctness or suitability for consumers intended use and/or application. For this and other reasons, we assume no responsibility and expressly disclaim liability for loss, damage or expense arising out of any way connected with the handling, storage, use or disposal of the product.  This MSDS was prepared expressly for this product.  Use the materials only as directed.  If the product is used as a component of another product, the information contained within the MSDS may not be applicable.  If one could have any concerns with or problems understanding this MSDS form, please direct all questions to INFOTRAC, Chemical Emergency Resources System at 1(800) 535-5053.

Revised Date: 2/2/12 | Replaces Date: 2/4/04