ADM Material Safety Data Sheet

Attach 2 Spray

ISSUE DATE: 	October 2003 	REVISION DATE: 	June 2007

MATERIAL SAFETY DATA SHEET

Attach 2

Alginate Tray Adhesive

COMPANY DETAILS

COMPANY NAME: 	ADM – Australian Dental Manufacturing

ACN:  010 967 432

ADDRESS: 	3 Papps Crt Rewood Park SA 5097

TELEPHONE: 	08 82635585

FAX: 	08 82635598

EMAIL: 	admchief@bigpond.net.au

EMERGENCY TELEPHONE NUMBER: 	131 126

| PRODUCT INFORMATION |  |
| --- | --- |
| PRODUCT NAME: Attach 2 |  |
| UN Number | 1950 |
| Dangerous Goods Class | 2.1 |
| Hazchem Code | 3[Y]E |
| Packaging Group | II |
| Poison Schedule | 5 |
| Product Use | Alginate Tray Adhesive |
| Pack Size | 215 mL Aerosol Spray Can |

Page 1 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

| COMPOSITION |  |  |
| --- | --- | --- |
| Chemical Composition HAZARDOUS INGREDIENTS: Hydrocarbon Solvent Mixture Dimethyl ether Resins (hydrocarbon based) | CAS number 64742-89-8 115-10-6 | Proportion 30-60% Propellant 10-40% |

| PHYSICAL AND CHEMICAL DATA |  |
| --- | --- |
| Appearance: | Reddish/Pink Coloured liquid |
| Odour | Hydrocarbon Solvent Odour |
| Melting Point: | Not Available |
| Boiling Point | -24.8°C (Propellant) >53°C (Solvent Component) |
| Vapour Pressure @ 25°C | <0.001 kPa (Propellant) |
| Specific Gravity | Unknown |
| Flash Point | -41°C (Propellant) |
| Flammable Limits | LEL: 1% (Solvent Component) UEL: 6% (Solvent Component) |
| Auto ignition Temperature | 235°C (Solvent Component) |
| Corrosiveness | Not Corrosive |
| Solubility | Miscible in Water Soluble in Hydrocarbon Solvents |

HEALTH HAZARDS IDENTIFICATION

This Product is classified as hazardous according to criteria of NOHSC due to its flammability.

Page 2 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

ACUTE EFFECTS

INGESTION:

EYE

SKIN:

INHALED

CHRONIC EFFECTS

•	May be harmful if swallowed. May cause nausea, irritation of the pulmonary system. Product may affect central nervous system.

•	If greater than approximately 60-70mL (1mL/kg body weight for average person) is swallowed symptoms such as weakness, dizziness and unconsciousness may occur.

•	If propellant enters mouth it may cause frost bite and freeze burns.

•	May cause mild irritation to eyes.

•	Propellant vapours are not expected to cause any irritation however if the propellant contacts the eye frost bite or freeze burns may cause permanent damage to the eye.

•	Repeated or prolonged exposure to skin may cause irritation due to defatting effect on skin.

•	Propellant may cause frost bite or freeze burns. Blistering with inflammation and pain may occur.

•	May cause irritation of respiratory tract, headaches, dizziness, loss of coordination nausea and other effects of the central nervous system.

•	Propellant is highly volatile. May cause irritation to eyes, nose or respiratory tract.

•	Inhalation of high concentrations may cause asphyxiation by displacement of oxygen from the breathing atmosphere. It may cause central nervous system depression such as dizziness, drowsiness, headache, mood disturbances, mental confusion and similar narcotic symptoms but no long term effects.

•	Signs of asphyxiation will be noticed when oxygen is reduced to below 16% and may occur in four stages. Symptoms include reduced alertness, fatigue, dizziness, headache, mental confusion, poor coordination.

•	Unconsciousness leading to central nervous system injury and possibly death will occur when the atmospheric oxygen concentration is reduced to about 6% to 8% or less. As this product is packaged in small containers exposure to high concentrations is unlikely unless intentional inhalation.

•	Repeated long term exposure to the product may cause internal injury to the following organs; lungs, liver, kidneys, nervous system. No studies have been conducted for long term effects, cancer or reproductive hazard.

Page 3 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

FIRST AID

INGESTION:

EYE:

SKIN:

INHALED:

•	Do not induce vomiting because of risk of aspiration into the lungs. Do not give anything by mouth to an unconscious person seek urgent immediate medical attention.

•	For minor ingestion rinse mouth and give water to drink.

Watch for toxic symptoms.

•	Open eye lids to irrigate the eye with copious amounts of water for 15 minutes.  If eyes continue to be affected seek immediate medical attention.

•	Remove contaminated clothing immediately.

•	Clothing frozen to the skin should be thawed then washed using lukewarm water. Seek medical attention.

•	If inhaled, immediately remove person to fresh air until fully recovered.  If person is not breathing, open airway and administer CPR (artificial respiration). Administer oxygen if breathing is difficult. Seek urgent medical attention.

Page 4 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

| EXPOSURE CONTROLS/PERSONAL PROTECTION |  |
| --- | --- |
| In accordance with the criteria of NOHSC this Product is classified as hazardous. |  |
| TWA (Solvent Component) | 400ppm NOHSC |
| STEL (Solvent Component) | 500ppm NOHSC |
| Personal Protection | Use gloves and safety glasses when handling the product. Use product in well ventilated area – the use of masks is not normally needed. If misting in confined areas and there is a risk of inhalation, wear approved respiratory protection. Respirators should comply with AS 1716 and be used in accordance with AS 1715. |
| Engineering Controls | • Follow procedures to avoid static discharges. Use non-spark tools and flameproof equipment. • The Product contains flammable solvents, therefore when using the product in a confined area it is recommended that the above exposure limits. • Provide ventilation to control exposure level below airborne exposure limits. Avoid skin and eye contact. It is recommended to wear, safety glasses and solvent resistant gloves when handling the product. Always wash hands before eating and drinking. • If there is risk of inhalation wear approved respiratory protection. Respirators should comply with AS1716 and should be used in according with AS1715. • TWA is the time-weighed average airborne concentration over an eight-hour working day, for a five day working week over an entire working life. This concentration should neither impair the health, or not cause undue discomfort to, nearly all workers according to current information. • STEL (Short Term Exposure Limit) is the average airborne concentration over a 15 minute period which should not be exceeded at any time during a normal eight hour work day. |

Page 5 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

| COMBUSTIBILITY |  |
| --- | --- |
| FIRE HAZARD & EXPLOSION: COMBUSTION PRODUCTS | • Product contains highly flammable solvents and highly flammable liquefied gas under pressure. • Isolate from sources of heat, naked flames or sparks. • If leakage of a container occurs evacuate the area. The container may release flammable vapours at ambient temperatures and readily forms a flammable mixture with air. Vapours are heavier than air and may travel long distances to a point of ignition and flash back. If practical remove the container outside and/or cut off gas supply if safe to do so – do not endanger life. • Container may explode in heat or fire. Use water to keep the container cool. Notify emergency services. • For small fires use dry chemical, CO2 or foam extinguishers • The ingredients will breakdown under fire conditions. Decomposition products are Carbon Dioxide, Water Vapour, traces of Carbon Monoxide. Fumes, smoke and Carbon Monoxide can be formed during incomplete combustion. • Fire fighters may need self-containing breathing apparatus. |

ACCIDENTAL RELEASE MEASURES

•	Avoid accidents, clean up immediately.

•	Wear protective equipment to prevent skin and eye contact.

•	Wipe up small spills with paper toweling and dispose to bin. Disposal of waste must be in accordance with the Local Waste Disposal Authority.

•	Prevent runoff to drains leading to waterways.  Wash contaminated area liberally with detergent and water.

•	For large spills or leaks in confined areas explosive conditions may occur, restrict persons not wearing protective equipment from the area of spill or leak until cleanup is complete.

•	Ventilate the area of spill or leak.

•	Water spray may be used to disperse vapours.

•	Absorb liquids in vermiculite, dry sand, earth or inert material and deposit in an appropriate waste disposal container.

•	For large spills or leaks it may be necessary to dispose of the product as a Hazardous waste.

Contact your local EPA or authority for specific recommendations.

Page 6 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

HANDLING & STORAGE

•	Product to be handled only by qualified staff in dental offices/laboratories.

•	See product instructions for safe use

•	Store in a cool well ventilated place out of direct sunlight.

•	Store away from strong oxidizing agents.

•	Store below 25°C keep away from sources of heat.

•	Protect containers from physical damage, check regularly for leaks.

•	Avoid contact with skins and eyes.

•	Do not store together with food.

Risk Phrases

R11:  Highly Flammable R26:  Harmful by inhalation R36:  Irritating to eyes

R38:  Irritating to skin

Safety Phrases

S2:  Keep out of reach of children

S9: Keep container in well ventilated place

S16:  Keep away from sources of ignition – no smoking

S23:  Do not breath gas/fumes/vapour/spray S24/25:  Avoid contact with skin and eyes S29: Do not empty into drains

S38: In case of insufficient ventilation use suitable respiratory equipment

S62: If swallowed, do not induce vomiting; seek medical attention immediately and show the container or label.

STABILITY & REACTIVITY

REACTIVITY CONDITIONS TO AVOID:

INCOMPATIBLES:

•	Stable at normal temperatures and pressure.

•	Avoid sources of heat and ignition.

•	Avoid contact with incompatible materials.

•	Reacts with strong oxidizing agents.

Page 7 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

TOXICOLOGICAL INFORMATION

SYMPTOMS: No adverse health effects are expected if this product is handled in accordance with this Material Safety Data Sheet and the Product label/instructions. If the product is spilled or mishandled, symptoms noted in section “Acute Effect” may occur: Long Term Effects: No specific long-term data is available for this product. Long-term occupational skin exposure to the solvent components has reportedly caused dermatitis and cracking of the skin due to defatting of the skin. No other long-term effects have been reported for the other ingredients.

ECOLOGICAL INFORMATION

• Spillages from the small packaging of this product are unlikely to penetrate soils. The solvents used in this product are volatile and will partition to the atmosphere. • The resin component will harden quickly and remain inert with no water-soluble material to leach into the environment. • Unlikely to cause long term adverse effects in the environment. This material is not expected to bioaccumulate. • As the solvent component is likely to vaporize quickly it is unlikely to reach waterways to cause long-term effect to the aquatic environment. No specific toxicity data to aquatic organisms is available for this product. • The ingredients are completely biodegradable. • No long-term persistent effects are expected. Due to the small package size spillages are unlikely to be a nuisance to the environment. If released to penetrate soils the resin component will be absorb on the soil and become immobile. • The product is insoluble in water and unlikely to reach waterways.

DISPOSAL CONSIDERATIONS

Dispose containers thoughtfully to landfill.

Empty containers represent a fire hazard as they may contain flammable product residues and vapour. Never weld or solder empty containers.

Page 8 of 8

ADM Material Safety Data Sheet

Attach 2 Spray

TRANSPORT INFORMATION

Classified as a highly flammable liquid gas for transport (ADG, UN, IATA)

UN NUMBER 	1133

CLASS 	2.1

SUBSIDIARY RISK: 	Nil HAZCHEM CODE: 	 3[Y]E Packaging Group 	 II

REGULATORY INFORMATION

From information available, this Product is not classified using the criteria of NOHSC.

CONTACT POINT

| CONTACT: | Business Hours: | 08 82635585 |
| --- | --- | --- |
|  | Mobile: | 0419665074 |
|  | Emergency | 131 126 |

This MSDS summarises our best knowledge of the health and safety hazard information of this

Product and how to safely handle and use the product in the workplace.

Each user should read this MSDS and consider the information in the context of how the Product will be handled and used in the workplace including its use in conjunction with other products.

If clarification of any term in this MSDS or further information is required to ensure safe use of this

Product can be made the user should contact ADM – Australian Dental Manufacturing

OTHER INFORMATION

This MSDS is prepared in accordance with the document “National Code of Practice for the

Preparation of Material Safety Data Sheets”, 1994, Worksafe Australia. Contact Points:	Australia

Police & Fire: 	000

National Poisons Information Centre: 	131126  (anywhere in Australia)

READ LABEL CAREFULLY BEFORE USING PRODUCT

Page 9 of 8