| 1. Product identification/preparation/company undertaking |  |  |  |  |
| --- | --- | --- | --- | --- |
| Trade name: |  | ## LC HAFTOPAQUE 1K |  |  |
| Use of the preparation: |  | Opaquer |  |  |
| Manufacturer/supplier: |  | Ultimate Dental, 25/632-642 Clayton Road Clayton Sth Vic 3169 |  |  |
| Emergency information: |  | (03) 9532 1799 or 131126 |  |  |
| 2. Hazards identification |  |  |  |  |
| Risk identification: |  | Highly flammable, Irritating |  |  |
| Special risks for human beings and environment: |  | Irritating to eyes, respiratory system and skin. # May cause sensitization by skin contact |  |  |
| Classification: |  | Classification corresponds to actual EC guidelines, however completed with details from technical literature and company data. |  |  |
| 3. Composition/Information of Ingredients |  |  |  |  |
| Chemical characterization: |  | Methacrylates, pigments, initiators |  |  |
| Description: |  |  |  |  |
| Hazardous components: |  |  |  |  |
| EC-No.: 201-297-1 CAS-No.: 80-62-6 | Name Methyl methacrylate |  |  | Letter R-classification ### F, Xi R 11-37/38-43 |
| 4. First-aid measures |  |  |  |  |
| General information: |  |  |  |  |
| after skin contact: |  | Wash off with plenty of water. In case of skin irritation, consult a physician. |  |  |
| after inhalation: |  | Fresh air. Consult a physician if feeling unwell. |  |  |
| after eye contact: |  | Rinse with plenty of water with the eyelid held wide open and consult an ophtalmologist |  |  |
| after swallowing: |  | Rinse mouth immediately and drink plenty of water. Seek medical advice if necessary. |  |  |
| 5. Fire fighting measures |  |  |  |  |
| Extinguishing media: |  | CO2, Water, powder, foam. |  |  |
| Protective equipment: |  | Chose in dependence to surrounding fire. |  |  |

| 6. Accidental release measures |  |
| --- | --- |
| Personal precautions: | Do not inhale vapours/aerosols. Ensure supply of fresh air in enclosed rooms. |
| Environmental precautions: | Do not allow to enter drains/ground-water. |
| Methods for cleaning up: | Take-up with liqid-adsorbent material. Forward for disposal. Clean up affected area. |
| Additional information: | Ventilate affected area. |
| 7. Handling and storage |  |
| Handling: |  |
| Handling procedures: | Keep containers closed. Use only in well ventilated areas. |
| Recommendation for fire and explosion protection: | Keep away from sources of ignition. Take measure to prevent electrostatic charge |
| storage: | Store at dry place (2 – 8 °C) |
| Requirements at storerooms and containers: | Tightly closed in a well ventilated place. Protected from light. |
| Requirements for storage with other products: | Store away from food stuffs. |
| Additional storage conditions: | -- |
| 8. Exposure controls/personal protection |  |
| Additional information for technical plants: | None. |
| Components with limits of values to be supervised at the workplace: CAS-Nr. Name art value unit [80-62-6] Methyl methacrylate TRGS 900 (Germany) 210 mg/m3 resp. 50 ml/m3 |  |
| Additional information: |  |
| Personal protective equipment: |  |
| General measures of protection and hygiene: | Protective clothing needs to be selected specifically for the workplace, depending on concentrations and quantities of the handled substances. |
| respiration: | Required when vapours/aerosols are generated. Filter A |
| hands: | Protective gloves (butyl-rubber). |
| eyes: | Safety glasses. |

| 9. Physical and chemical properties |  |  |
| --- | --- | --- |
| Appearence |  |  |
| Form: | Liquid. |  |
| Colour: | Colourless. |  |
| Odour: | Characteristic |  |
|  | Value Unit Method |  |
| Information on change in the physical state |  |  |
| Melting point/melting range: | n.a. °C |  |
| Boiling point/boiling range: | 101 °C |  |
| Flash point: | 10 °C DIN 51794 |  |
| Autoignation temperature: | No information available |  |
| Danger of explosion: | Lower 2.1 Vol % Upper 12.5 Vol % |  |
| Density: | 1,08 g/cm3 |  |
| Vapour pressure: | n.d. |  |
| Viskosity: | n.d. |  |
| pH: | n.a. |  |
| Solubility in/miscibility with: | Organic solvents |  |
| Water: | Sligtly soluble |  |
| Content of solvents: |  |  |
| Organic solvents: | None |  |
| Water: | --- |  |
| Content of solids: | None |  |
| 10. Stability and reactivity |  |  |
| Incompatibility with other substances: | # Warming > 50°C. Exposure to light |  |
| Hazardous discomposition products: | Not known. |  |
| 11. Toxicological information | allergic reation to methacrylate by chronic exposure to product are possible |  |
| Acute toxicity: | LD50 (rat/oral): 7872 mg/kg |  |
| Primary irritation: |  |  |
| skin: | irritating |  |
| eye: | irritating. |  |
| ingestion: | harmful. |  |
| sensitisation | Sensitising in case of skin contact possible |  |
| Additional toxikological information: | Liquid is irritant. |  |

| 12. Ecological information |  |
| --- | --- |
| General information: | Do not allow to enter drains/canalisation/ground-water |
| Classification of water endangerment: | WGK = 1 (slightly polluting substance). |
| 13. Disposal consideration |  |
| Product: |  |
| Recommendation: | Dispose of waste according to applicable local, state, and federal regulations. |
| Disposal key number: |  |
| Uncleaned packaging: | Handle contaminated packaging in the same way as the substance itself. |
| Recommendation: |  |
| 14. Transport information Land transport ADR, RID | UN 1247 METHYLMETHACRYLAT, MONOMER, STABILISIERT, 3,II |
| Sea transport IMDG | UN 1247 METHYL METHACRYLATE, MONOMER, STABILIZED, 3, II EmS: F-E S-D |
| Air transport ICAO-TI/IATA-DGR | UN 1247 METHYL METHACRYLATE, MONOMER, STABILIZED, 3, II The transport regulations are cited according to international regulations and in the form applicable in Germany. Possible national deviations in other countries are not considered. |
| 15. Regulatory information |  |
| Classification according to EC-guidelines: | Xi Irritant F Highly flammable R 11- Highly flammable. 37/38 Irritating to respiratory system and skin. R 43 May cause sensitization by skin contact. S 24 Avoid contact with skin. S 37 Wear suitable gloves |
| National Prescriptions: | None |
| 16. Other information: |  |
| Changes compared with the previous version: | The above information is based on our present day knowledge and relates solely to the safety requirements of the product. The data do not signify any warranty with regards to products properties. However users of the product should satisfy themselves that the information given is sufficient and correct for their specific circumstances of use. |
| Preparation by: | Ultimate Dental 03 9532 1799 |