1          chemical product and company information

1.1       trade name

Die Spacer – all colours

Danger symbols:

1.2	details of manufacturer/shipper DFS-DIAMON GmbH Ländenstraße 1

93339 Riedenburg

Tel. Ultimate Dental 03 9532 1799    Poisons Information: 13 11 26

2          composition/information on ingredients

chemical characteristics                                              agent     preparation

| CAS – number | name | % - content | R- phrase |
| --- | --- | --- | --- |
| 67-63-0 | Isopropyl alcohol | 45 | 11 |
| 107-98-2 | Propylene glycol-1-methyl ether | 10 | 10 |
| 67-64-1 | Acetone | 45 | 11 |

3          hazards identfication

hazard symbol:          R 11: easily inflammable

4          first aid measures

after inhalation:

None

after skin contact:

Wash skin with soap and water after contact. Take off contaminated and soaked clothes. Do not use solvent or thinner.

after swallowing:

In case of swallowing arrange immediately for medical treatment. Keep the concerned person calm.

after eye contact:

Irrigate eyes immediately for several minutes with plenty of water and consult physician if symptoms persist

5          fire fighting measures

suitable extinguishing agents:

CO2, extinguishing powder, foam or water dust

for safety reasons, unsuitable extinguishing agents

Water jets

special equipment for fire fighting

Wear self-contained-breathing-apparatus

6          accidental spill release measures

personal precautions:

Wear protective equipment. Keep unprotected persons at distance. Use respiratory protection in case of dust.

environmental protection measures:

Do not discharge into the sewage system, surface water, groundwater

methods for cleaning up:

Sweep up spilt product with wax impregnated sawdust. Avoid dusting. Clean floor and soiled objects with water and cleansing agent. The absorbed material must be disposed of in accordance with local, state and federal regulations.

7          handling and storage

7.1        handling

instructions for safety handling

No special measures are required if used properly.

7.2        storage

requirements for storage rooms and containers:

Protect against sun and heat above 50°C. Keep containers tightly sealed. Do not store close to food.

| 8 exposure controls and personal protection |  |  |  |
| --- | --- | --- | --- |
| 8.1 exposure control parameters |  |  |  |
| CAS – number | name | value | unit |
| 67-63-0 | Isopropylalcohol | 980 | ppm |
| 107-98-2 | Propylene glycol-1-methyl ether | 375 | ppm |
| 67-64-1 | Acetone | 1200 | ppm |

8.2        personal protection equipment

general protection and hygienic measures:

Keep away from food and beverage. Wash the skin thoroughly after work and before breaks.

respiratory protection: None

hand protection:

Wear safety gloves made of PVC.

eye protection:

Wear protection goggles

body protection:

Wear antistatic clothing made of natural fibres (f. e. cotton)

| 9 physical and chemical characteristics |  |  |
| --- | --- | --- |
| material state: liquid | colour: red, blue, silver, gold, clear | odour: aromatic |
|  | value | unit |
| flash point | < 21 | °C |
| ignition temperature | 200 – 300 | °C |
| density (at 20 °C) | 0,880 | g/cm³ |
| lower explosion limit | 0,8 | Vol % |

10        stability and  reactivity

hazardous decomposition products:

No decomposition will occur in case of appropriate usage and storage

11        toxicological information

A narcotic effect is possible in case of high concentration due to the solvent.

12        ecological information

general instructions

None

13        disposal considerations

recommendation:

Must be disposed of by an authorised waste/disposal company in accordance with local state and federal regulations, e. g. suitable incineration plant, due to emission of hydrogen halide. Contaminated packing must be emptied completely.

waste key code:            55502  paint

14        transport regulations

Transport only according to the transport instructions for street (ADR), rail (RID), ocean (IMGD and air

(ICAO/IATA)

country transport ADR/RID and GGVS/GGVE (transborder / domestic)

ADR / RID – GGVS / E – class:   3                                      figure / letter: 5B

name of the good: Die Spacer

14.1      ocean transport IMDG / GGV – sea

IMDG / GGV sea – class:                                                     UN – Nr. 1263

14.2      air transport ICAO TI and IATA – DGR

packing class:                               II

further declarations none

15        regulatory insturction

15.1      EEC marking and labeling

R – phrases

R 11     easily inflammable

S – phrases

S 2        keep out of reach of children

S 9        store container in a well ventilated room

S 16     Protect against sun and heat above 50 °C

water hazard class:

WHC 2 (self classification)

16        other details

16.1 further information: The details are based on our current state of knowledge, however, they do not constitute any assurance of product characteristics and cannot be the basis of any legal contractual relations.