MATERIAL SAFETY DATA SHEET

PRODUCT NAME:		Dental wax

OTHER NAMES:		Peripheral

Sprue

Bite

Orthodontic

PHYSICAL PROPERTIES:

Melting Point:			55 - 65C

Flash Point:			>200C

FIRE/EXPLOSION HAZARD:

No explosion hazard from these waxes under normal conditions.  Contact with water if temperature exceeds 100C may cause boil over.

EXTINGUISHING MEDIA:	Foam, carbon dioxide, dry chemical.

DO NOT USE WATER ON BURNING WAX.

HEALTH EFFECTS:

Swallowed:			Non Toxic.  Small quantities will pass undigested.

Eyes:				Low hazard unless molten.  When molten burns may occur.

Skin:				Low hazard unless molten.  When molten burns may occur.

Inhaled:	Nil under normal conditions.  Hazard if continued exposure

to fumes when molten.

FIRST AID:

Swallowed:	Provide drink of water.  Do not induce vomiting.

Eyes:	Non irritant.  Molten wax may cause burns.  Seek medical

advice.

Skin:	Contact with molten wax may cause burns.  Seek medical

advice.

Inhaled:	Move victim to fresh air if overcome by fumes.  Seek

medical advice.

CONTROL MEASURES:

Exposure:	Good ventilation when working with molten wax (<2mg/m3 concentration) is required.

PERSONAL PROTECTION:

Eye:	Goggles or safety glasses should be worn when working with molten wax.

Skin:	Gloves and apron should be worn.

Inhaled:	Face mask should be worn when working with molten wax.

SPILLS AND LEAKS:

Uncontaminated wax may be scraped and re-used.  Contaminated material may be scraped and disposed of

in accordance with local, state and federal regulations.

STORAGE:

Waxes should be stored in a cool dry place away from strong oxidants, extreme heat and naked flames.

DISCLAIMER:

This information is provided to the best of our knowledge and recommendations made are for the users

consideration.  The user should ensure that they are suitable for their particular uses and applications.

The information should not be considered as guaranteeing specific properties as conditions of use are

beyond our control.